Subcontractor´s certificate – Plastic and rubber

Certificate refers to component/product:

Click to fill.

2.1      Mandatory material requirements

| 2.1.1 SVHC/ECHA’s candidate list |  |  |  |
| --- | --- | --- | --- |
| No chemical substances listed on ECHA's candidate list of SVHC substances may occur in concentrations above 0.1% by weight, see https://echa.europa.eu/candidate-list-table 0.1% by weight refers to chemical products or articles that are included or used in the manufacture of the furniture/product (i.e. each individual part of an item), not 0.1% of an assembled piece of furniture, see guidance document here: https://echa.europa.eu/documents/10162/2324906/articles_en.pdf |  |  |  |
| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |

2.1.4     Plastic and rubber

This section includes plastic parts, rubber parts, padding materials that contain plastic (eg cellular plastic or polyurethane foam), woven plastic, artificial leather and plastic coating on textiles and leather. Woven plastic/artificial leather marked with Oeko-Tex 100 (product class I, II) meets all the requirements except for 2.1.4.3 which must be verified separately.

| 2.1.4.1 Flame retardants in plastic/rubber |  |
| --- | --- |
| None of the flame retardants below have been actively added or that the levels do not exceed 0.1% by weight, must be available. Electronics (e.g. electric motors and electrical cables) are exempted. Small plastic parts <100 g (eg screws, pins and fasteners) are excluded from this requirement. |  |
| Polybrominated biphenyls (PBBs) | CAS: 59536-65-1 |
| Oktabromdiphenylether (oktaBDE) | CAS: 32536-52-0 |
| Tris (2,3-dibrompropyl) phosphate (TBPP) | CAS: 126-72-7 |
| Tris (1-aziridinyl) phosphine oxide (TEPA) | CAS: 5455-55-1 |
| Tris (2 chlorethyl) phosphate (TCEP) | CAS: 115-96-8 |
| Tris(1,3-dichloroisopropyl)phosphate (TDCPP) | CAS-nr: 13674-87-8 |

| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |
| --- | --- | --- | --- |

2.1.4.2 Softeners/phthalates in plastic/rubber

No phthalates/softeners classified as hazard classification H340, H350, H360 have actively been added and that the measured level does not exceed 0.1% by weight/substance and component, must be available. Small plastic parts (eg screws, pins and fasteners) are excluded from this requirement.

| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |
| --- | --- | --- | --- |

2.1.4.2 Softeners/phthalates in plastic/rubber

For school environment, documentation for the included plastic/rubber must be available which proves that phthalates/softeners below have not been actively added or that the measured value amounts to a maximum of 0.1% by weight per substance and detail. Small plastic parts <100 g (eg screws, pins and fasteners) are not covered by the requirement. - DNOP (CAS-nr: 117-84-0) - DIDP (CAS-nr: 68515-49-1) - DINP (CAS-nr: 68515-48-0)

| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |
| --- | --- | --- | --- |

2.1.4.3 PVC

Included plastic parts including coated fabrics/artificial leather, do not contain PVC must be available. Electrical components (e.g. power cables) and hospital/urine textile or coated fabric/artificial leather on furniture in healthcare environment where regular disinfection with alcohol is required are excluded from this requirement. Small plastic parts <100 g (eg screws, pins and fasteners) are excluded from this requirement.

| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |
| --- | --- | --- | --- |

2.1.4.4 Pigments in plastics/rubber

No pigments or additives based on lead, cadmium, tin, chromium VI or mercury have been actively added or that the levels do not exceed 0.01% by weight per component. Small plastic parts <100 g (eg screws, pins and fasteners) are excluded from this requirement.

| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |
| --- | --- | --- | --- |

2.1.4.5 Short-Chain Chlorinated Paraffin (SCCPs) in plastic/rubber

No short-chain chlorinated paraffins (SCCPs) have been actively added or are included, must be available. Content must not exceed 0.01% by weight as measured value per component. Small plastic parts <100 g (eg screws, pins and fasteners) are excluded from this requirement.

| Does the product meet the above requirements? | Yes☐ | No☐ | Not relevant☐ |
| --- | --- | --- | --- |

This certifies that the component/product to which the certificate relates meets the above requirements according to Möbelfakta's requirement specification 2023-04-01 ver 15.

Supplier:		Klicka och ange

Place:		Klicka och ange

Date:		Klicka och ange.

Signature: ________________________________________

Print name: 	Klicka och ange.